Business Requirements:

- Controller for parking access which support multiple communication protocol

Parking Access Control Board Peripherals:

- Short-Range: Wiegand/RS485 (2 GPIO + GND)/RS232 (4 GPIO + GND).

- QR Reader: USB.

- Mic/Speaker  Microphone and Speaker.

- Intercom  Network/Ethernet.

- ANPR Camera  Ethernet.

- Ticket Printer  USB.

- Activation LED  1 GPIO.

- Reserved GPIO (1-3)- Status – RGB LED.

- Inductive Loop  GPIO (3 IN + GND).

- Barrier Control  GPIO (5 Pin “Relay” + Vcc + GND).

- CCTV pin hole camera  stripe block or USB.

- Display –> ??

| CPU | Quad-core Arm Cortex A76 processor @ 2.4GHz |
| --- | --- |
| RAM | Min. 4GB LPDDR4-3200 SDRAM and extendable up to 32 or higher |
| ROM | Min. 32GB eMMC Flash memory |
| Storage | SD Card up to 512 GB. |
| GPU | NVIDIA (Optional) |
| USB | Min. 3 (Extra 1) |
| Wiegand | Min. 1 (Extra 1) |
| RS232 | Min. 1 (to be confirmed) – 4 GPIO |
| RS485 | Min. 1 (to be confirmed) – 3 GPIO |
| Relay | Min. 5 (Extra 3) – 5-8 GPIO |
| Ethernet | Yes |
| Wi-Fi | Optional |
| GSM/SIM Card | Mandatory |
| Bluetooth | Bluetooth 5.0, BLE (Optional) |
| GPIO | Min. 13-28 (Externally or embeded. Provide recommendations) |
| Temp. | -20C – 65C |
| Humidity | 0%-98% (noncondensing) |
| Dimensions | 15.00 x 15.00 cm (Recommendation for smaller) |
| HW Status LED | 1 x RGB for booting and HW status. |
| Provition LEDs | 6 x RGB for device activation (Optional) |
| Mic | Optional |
| Speaker | Optional |
| Intercom action Button | 1 GPIO (Optional) |
| Display | ?? |
| HDMI | Optional |
| Action Buttons | 4 GPIO (Optional) |
| Receipt Printer | USB (Optional) |
| Sensors | Temp + Humidity |